Standard Operating Procedure: Equipment Cleaning and Swab Sampling Between Product Changeovers

Document Number: SOP-MFG-221

Version: 4.0

Effective Date: 01/20/2026

Department: Manufacturing - Solid Dose

1.0 PURPOSE

This procedure defines the cleaning, swab sampling, and residue-limit verification

required on the V-blender and tablet press train before a different drug product is

introduced to the equipment train, so that cross-contamination between products is

prevented and the equipment is released for the next changeover with objective evidence.

2.0 SCOPE

This procedure applies to all equipment in Suite 2 shared between more than one drug

product, and to all Manufacturing Operators, Cleaning Verification Technicians, and

Quality Assurance staff involved in changeover cleaning. It does not apply to dedicated

single-product equipment, which follows SOP-MFG-104.

3.0 DEFINITIONS

Maximum Allowable Carryover (MACO): The calculated maximum quantity of the previous product's active ingredient that may remain on shared equipment surfaces without posing a safety risk to the next product.

Swab Sample: A defined 25 square centimeter surface area sampled with a wetted swab and submitted for residue analysis.

Clean Hold Time: The maximum 72-hour interval during which cleaned equipment may sit idle before requiring re-cleaning prior to use.

Rinse Sample: A sample of the final rinse water collected to verify residue and detergent removal on surfaces a swab cannot reach.

4.0 RESPONSIBILITIES

- Manufacturing Operators: Perform the cleaning cycle and complete the cleaning log

- Cleaning Verification Technicians: Collect swab and rinse samples per this procedure

- Quality Assurance: Review results against the MACO limit and release equipment for use

- Laboratory: Analyze swab and rinse samples and report results within 48 hours

SAFETY WARNINGS

WARNING: Do not enter the blender bowl until lockout/tagout has been applied to the drive

motor and verified with a zero-energy check.

CAUTION: Cleaning solvent is flammable. No open flame or spark-producing tools are

permitted within 10 feet of the cleaning station while solvent is in use.

WARNING: Respiratory protection rated for the previous product's occupational exposure

band must be worn until swab results confirm the surface is below the MACO limit.

DANGER: Never bypass the tablet press guard interlock to reach cleaning solvent into the

compression zone while the turret is capable of rotating.

5.0 PROCEDURE

5.1 Verify Lockout and Disassembly

Apply lockout/tagout to the equipment train per SOP-EHS-002 and verify zero energy state.

Disassemble removable product-contact parts, including the blender bowl liner, press

tooling, and dust collection ductwork, per the equipment-specific teardown instruction.

5.2 Perform the Detergent Wash Cycle

Wash all product-contact surfaces with the qualified alkaline detergent at 55 degrees

Celsius for a minimum of 15 minutes of contact time, using a soft nylon brush on

hard-to-reach surfaces such as the blender shaft seal.

a. Confirm the detergent concentration is between 1.0 and 1.5 percent by volume

b. Confirm wash water temperature with the calibrated thermometer before starting the timer

5.3 Perform the Final Rinse

Rinse all surfaces with purified water until the conductivity of the rinse effluent is at

or below 5 microsiemens per centimeter, confirmed with the in-line conductivity meter.

Collect a rinse sample from the final rinse of the blender bowl for laboratory analysis.

5.4 Visually Inspect All Surfaces

Inspect all product-contact surfaces under white light of at least 500 lux for visible

residue, discoloration, or detergent film. Any surface showing visible residue fails

inspection and must be re-cleaned before swabbing.

5.5 Collect Swab Samples

Collect swab samples from the six designated locations shown on the equipment cleaning

map, including the blender shaft seal, the discharge valve, and the press die table. Each

swab covers a 25 square centimeter area using the validated swabbing technique.

a. Use a new sterile swab and glove pair for each location

b. Label each swab vial with the location code, lot number, and date

5.6 Submit Samples and Apply the Clean Hold Label

Submit the swab and rinse samples to the laboratory with a completed chain-of-custody

form. Apply the green Clean Hold label to the equipment, recording the cleaning

completion time; the 72-hour clean hold clock begins at this timestamp.

5.7 Review Laboratory Results Against the MACO Limit

Quality Assurance reviews the reported residue result for each swab location against the

calculated MACO limit of 4.0 micrograms per 25 square centimeters. Any result exceeding

the MACO limit fails the cleaning verification and equipment must be re-cleaned and

re-sampled.

5.8 Release the Equipment for the Next Product

Once all six swab results and the rinse sample pass, Quality Assurance signs the

Equipment Release Log, records the next product and lot number authorized to use the

equipment, and removes the Clean Hold label, replacing it with the equipment status tag.

5.9 Reassemble and Verify Clean Hold Time at Use

Reassemble the equipment using only the parts identified on the teardown record. Before

production start, confirm the elapsed time since the cleaning completion timestamp does

not exceed the 72-hour clean hold limit; if exceeded, the equipment must be re-cleaned.

REFERENCES:

- SOP-EHS-002: Lockout/Tagout Procedure

- SOP-MFG-104: Dedicated Equipment Cleaning

- SOP-QC-045: Swab and Rinse Sample Analysis

- Company Cleaning Validation Master Plan, Section 6

RECORDS:

- Equipment Cleaning Log (Form MFG-221-A)

- Equipment Release Log (Form MFG-221-B)

REVISION HISTORY:

Version 4.0 (01/20/2026): Lowered MACO limit following updated toxicology assessment

Version 3.1 (05/12/2025): Added rinse sample requirement for the blender bowl

Version 3.0 (09/01/2023): Initial cross-contamination risk assessment incorporated